AIR TRAFFIC MANAGEMENT (ATM)

CONTINGENCY PLAN APPLICABLE TO

MOGADISHU FLIGHT INFORMATION REGION (FIR)

PREPARED BY

Civil Aviation Caretaker Authority for Somalia (CACAS)

TABLE OF CONTENTS

Page Number

Forward										ii

Record of Amendments									iii

Objective										1

States and Flight Information Regions Affected						1

Management of the Contingency Plan							2

Contingency Routes Structure								3

Air Traffic Management and Contingency Procedures					4

Transfer of control and coordination of traffic						8

Volcanic Ash Contingency Plan								8

Pilot and Operator Procedures								8

Communication Procedures								11

Aeronautical Information Support Services						11

Search and Rescue									12

Responsibility of the accredited ICAO Regional Office (ESAF)				12

--------

Appendix A -	Contact details for all concerned States, IATA and accredited

ICAO Regional Office

Appendix B - 	Central Coordinating Committee contacts

Appendix C -	Contingency Routes Structure during total unavailability of the Mogadishu FIR or associated air traffic services.

Appendix D - 	Contingency Route Structure – Chart

Appendix E -	Contingency Radio Communication Frequencies

Appendix G - 	Volcanic Ash Contingency Plan

FOREWORD

This Contingency Plan will come into effect as determined by the Civil Aviation Caretaker Authority for Somalia (CACAS), who is the authority responsible for civil aviation operations in Somalia.

The Plan has been prepared in coordination with the International Civil Aviation Organization (ICAO) to meet the provisions of ICAO Annex 11 ─ Air Traffic Services Chapter 2 (2.30), to provide for the safe and orderly continuation of international flights through the Mogadishu FIR.

The Plan has been developed in coordination with the Somalia airspace management stakeholders (CACAS, ICAO,) and with the close co-operation and collaboration with IATA and the civil aviation authorities and air navigation service providers (ANSPs) responsible for the adjacent FIRs.

The Plan will be activated by promulgation of a NOTAM issued by Somalia (CACAS) International NOTAM Office (NOF) as far in advance as is practicable. However, when such prior notification is impracticable for any reason, the Plan will be put into effect through notification by the designated authority as authorized by CACAS or the United Nations. In the event that this is also not practical, notification may be made by ICAO in accordance with arrangements made with CACAS.

Arrangements have been made with civil aviation authorities responsible for adjacent airspaces, and action on their part in the event of activation of the Plan will be in accordance with operational Letters of Agreement (LOAs) established between Somalia (CACAS) and adjacent States concerned. Aircraft flying through the Mogadishu FIR during activation and operation of the Mogadishu Contingency Plan are expected to comply with the requirements of this Plan and to cooperate with other airspace users as necessary for continued safety of air navigation.

It is to be understood that contingency arrangements that constitute a temporary deviation from the approved Regional Air Navigation Plan are subject to approval as necessary, by the President of the ICAO Council on behalf of the Council.

Proposed amendments to this plan shall be forwarded to:

Mr. Kemoitse J. Mosupukwa

Project Coordinator

Civil Aviation Caretaker Authority for Somalia

C/O ICAO ESAF Regional Office

P. O. Box 46294

Nairobi 00100

Kenya

Tel: + 254 20 7622785 (Office)

Tel: + 254 727 867 358 (Mob)

Fax: + 254 20 712 2340

Email: kemoitse.mosupukwa@icao.unon.org

RECORD OF AMENDMENTS

| Amendment Number | Effective Date | Date Entered | Entered By | Paragraph/ Reference |
| --- | --- | --- | --- | --- |

ATM CONTINGENCY PLAN FOR INTERNATIONAL FLIGHTS

TO TRANSIT THE AIRSPACE OF THE MOGADISHU FIR

Effective:    Date………………                                               Time……………..

1.	OBJECTIVE

  - This Air Traffic Management (ATM) Contingency Plan contains arrangements to ensure the continued safety of air navigation in the event of partial or total disruption of air traffic services in the Mogadishu FIR in accordance with ICAO Annex 11 ─ Air Traffic Services, Chapter 2, Section 2.30. The Contingency Plan provides the ATS procedures and contingency route structure using existing airways in most cases that will allow aircraft operators to transit the Mogadishu FIR.

  - This Contingency Plan does not address arrangements for aircraft arriving and departing at airports within Somalia, or for domestic flight operations within the territory of Somalia.

2.	STATES AND FLIGHT INFORMATION REGIONS AFFECTED

2.1			In the event that the Civil Aviation Caretaker Authority for Somalia activates this Contingency Plan, adjacent States and civil aviation authorities responsible for air navigation services in the adjacent FIRs will be notified in accordance with the Letters of Agreement established between Somalia (CACAS) and adjacent FIRs concerned. The adjacent States’ FIRs and ATS units directly affected by this Contingency Plan are as follows:

- State: Djibouti

Name of TMA: Djibouti TMA

Name of ATS Unit: Djibouti Approach/Tower

- State: Ethiopia

Name of FIR: Addis Ababa

Name of ACC: Addis Ababa Control

c)	State: Kenya

Name of FIR: Nairobi

Name of ACC: Nairobi Control

- State: India

Name of FIR: Mumbai

Name of ACC: Mumbai Control

- State: Seychelles

Name of FIR: Seychelles

Name of ACC: Seychelles Control

- State: Yemen

Name of FIR: Sana’a

Name of ACC: Sana’a Control

2.2		The contact details of the civil aviation authorities and organizations concerned are contained in Appendix A to this document.  These details will be kept up to date in accordance with Letters of Agreement and the Contingency Plan shall be updated accordingly.

3.	MANAGEMENT OF THE CONTINGENCY PLAN

3.1		The contingency measures set out in this Plan are based on known, foreseeable or probable impact of interruptions in ATS, caused by natural occurrences or other circumstances, which, in one way or another may partially or totally disrupt the provision of ATS and/or related support services in the Mogadishu FIR, or make the airspace unavailable or unsafe for use.

  - The following arrangements have been put in place to support management of the Contingency Plan in order to ensure that international flights may continue in a safe and orderly manner through the Mogadishu FIR.

Central Coordinating Committee

  - Whenever circumstances permit, as soon as practicable in advance of, or after a contingency event has occurred, the Project Coordinator of CACAS shall convene the Central Coordinating Committee (CCC) comprising representatives from:

- CACAS;

- ICAO ESAF Regional Office;

- ATS providers within Mogadishu FIR;

- UNSOA;

- Representative from the airlines;

- Other UN agencies and stakeholders considered necessary.

  - The Central Coordinating Committee (CCC) shall oversee the conduct of the Contingency Plan and in the event that the Mogadishu FIC premises are out of service for an extended period, make arrangements for and facilitate the temporary relocation of the Mogadishu FIC at the Temporary ACC and the restoration of ATS services. The terms of reference for the CCC will be determined by CACAS in consultation with applicable stakeholders and will be updated periodically and circulated to members of the CCC.

  - Contact details of the CCC members are provided in Appendix B to this document.

Mogadishu ATM Operational Contingency Group

  - The Mogadishu ATM Operational Contingency Group (MAOCG) will be convened by the CCC with a primary responsibility to oversee the day to day operations under the contingency arrangements, and coordinate operational ATS activities 24 hours a day, throughout the contingency period. The terms of reference of the AOCG will be determined by the CCC. The MAOCG will include specialized personnel from the following disciplines:

- Air traffic services (ATS)

- Aeronautical telecommunication (COM)

- Aeronautical meteorology (MET)

- Aeronautical information services (AIS)

- Communication, navigation and surveillance (CNS)

The tasks of the MAOCG shall include taking the following action:

- review and update of the Mogadishu Contingency Plan as required;

- keep up to date at all times of the contingency situation;

- organize contingency teams in each of the specialized areas;

- keep in contact with and update the ICAO ESAF Regional Office, operators and the IATA Regional Office;

- exchange up-to-date information with the adjacent ATS authorities concerned to coordinate contingency activities;

- notify the designated ATM organizations in Djibouti, Ethiopia, Kenya, India, Seychelles and Yemen of the contingency situation sufficiently in advance and/or as soon as practical thereafter; and

- issue NOTAMs according to the corresponding contingency situation related to this plan or as otherwise required. If the situation is foreseeable sufficiently in advance, a NOTAM will be issued at least 48 hours in advance.

4.	CONTINGENCY ROUTES STRUCTURE

  - In the event of disruption of air traffic services provided by Mogadishu FIC, contingency routes will be introduced to ensure safety of flight and to facilitate limited flight operations commensurate with the prevailing conditions. Existing ATS routes form the basis of the contingency routes to be used, and a flight level assignment scheme shall be introduced to minimize potential points of conflict and to limit the number of aircraft operating simultaneously in the system under reduced air traffic services, including surveillance.

  - The contingency route structure for international flights is detailed in Appendix C to this document. Additional contingency routes will be introduced as and when circumstances require, such as in the case of volcanic ash clouds formation.

  - In regard to domestic operations, if circumstances dictate, all flights shall be temporarily suspended until a full assessment of the prevailing conditions has been determined and sufficient air traffic services restored. A decision to curtail or restart domestic operations will be made by the CCC.

  - Aircraft on long-haul international flights and special operations (e.g. Search and Rescue (SAR), State aircraft, humanitarian flights, etc), shall be afforded priority for levels starting from FL290 and above.

  - International and domestic operators affected by the suspension of all operations from major airports in Somalia will be notified by the relevant airport authority when operations may be resumed, and flight planning information will be made available pertaining to those airports. International flights which have received such approval may be required to file flight plans via domestic routes to join international contingency routes.

  - International operators may elect to route around the Mogadishu FIR if this will satisfy operational requirements of their companies. In such instances, the contingency routes to be used will be provided by States providing air traffic services in the adjacent FIRs concerned.

# AIR TRAFFIC MANAGEMENT AND CONTINGENCY PROCEDURES

Reduced ATS and provision of flight information services (FIS)

  - During the contingency critical period, air traffic services (ATS), including air traffic control (ATC) may not be available, particularly with regard to availability of communications and surveillance services. In cases where such services are not available, a NOTAM will be issued by CACAS or adjacent ACCs, providing the relevant information, including an expected date and time of resumption of services. The Contingency Plan provides for limited flight information and alerting services to be provided by adjacent ACCs.

  - Flight information service (FIS) and flight monitoring will be provided by the designated ATS authorities for the adjacent FIRs on the contingency routes that enter their respective FIRs. A chart depicting the airspace arrangement is provided in Appendix D to this document.

  - The primary means of air-ground communication will be by HF radio except for aircraft operating automatic dependent surveillance (ADS) and controller/pilot data link communication (CPDLC) systems where this has been established and is fully operational. Where CPDLC has been established, this will become the primary means of communication, with HF as secondary. In the case of automatic position reporting, this will replace voice position reporting and CPDLC and/or HF will become the secondary means of communication.

# ATS Responsibilities

  - During the early stages of a contingency event, the Mogadishu FIC may become overloaded which may require tactical action to be taken in order to re-route aircraft on alternative routes that are not included in this Plan. Alternative routes will be designed to maximize the use of existing ATS route structures and communications, navigation and surveillance (CNS) services.

  - In the event that ATS cannot be provided in the Mogadishu FIR, a NOTAM shall be issued indicating the following, as a minimum requirement:

a)	time and date of the beginning of the contingency measures;

b)	airspace available for landing and overflying traffic and airspace to be avoided;

c)	details of the facilities and services available or not available and any limits on ATS provision (e.g., ACC, APP, TWR and FIS), including an expected date of restoration of services if available;

d)	flight level allocation scheme (FLAS) if different from those defined in Appendix C and D to this document;

e) 	information on the provisions made for alternative services;

f)	any changes to the ATS contingency routes contained in this Plan;

g)	any special procedures to be followed by neighbouring ATS units not covered by this Plan;

h)	any special procedures to be followed by pilots; and

i)	any other details with respect to the disruption and actions being taken that aircraft operators may find useful.

  - In the event that the Somalia (CACAS) International NOTAM Office is unable to issue the NOTAM, the (alternate) International NOTAM Office at Nairobi and/or Dar Es Salaam will take action to issue the NOTAM pertaining to the closure of airspace upon notification by CACAS or ICAO ESAF Regional Office. Sample NOTAMs are at Appendix E to this Contingency Plan.

Aircraft Separation

  - Aircraft separation criteria will be applied in accordance with the Procedures for Air Navigation Services-Air Traffic Management (PANS-ATM, Doc 4444) and the Regional Supplementary Procedures (Doc 7030).

  - The minimum longitudinal separation applicable will be 15 minutes.

  - The route structure provides for a minimum lateral separation of 50 nautical miles. In cases where this is, and for crossing routes, standard vertical separation shall be applied between all aircraft transiting the Mogadishu FIR.

Flight level restrictions

  - Where possible, aircraft on long-haul international flights shall be given priority with respect to the assignment of cruising levels.

Operational restrictions

  - VFR flights shall not operate in the Mogadishu FIR if there are extensive disruptions to ATS facilities, except in special cases such as State aircraft, medivac flights, and any other essential flights authorized by CACAS.

  - IFR General Aviation flights will receive a lower priority than all other flights and may be suspended depending on circumstances.

  - IFR commercial flights will receive a high priority together with State and medivac flights.

Other measures

  - Other measures related to the limited availability of airspace and the implementation of the contingency scheme within the Mogadishu FIR may be taken as follows:

    - Suspension of all VFR operations;

    - Delay or suspension of general aviation IFR operations; and

    - Delay or suspension of commercial IFR operations.

- Aircraft position reporting

  - Pilots will continue to make routine position reports in line with normal ATC reporting procedures. Pilots shall also use the IFBP VHF frequency 126.9 Mhz when making routine position reports.

- Procedures to be followed by Mogadishu FIC and adjacent ATS Units

  - Mogadishu FIC and adjacent ATS units will follow their emergency operating procedures and activate the appropriate level of contingency procedures in line with operational Letters of Agreement. These procedures shall include the following:

- the Mogadishu FIC, on determining that air traffic services may be reduced due to a contingency event, will inform pilots accordingly. In the event of incapacitation of the operations room/building, the appropriate emergency procedures will apply and time permitting, controllers will make an emergency evacuation transmission on the radio frequency or frequencies in use providing pilots with alternate means of communication;

- during the period when the contingency procedures are in effect, flight plan messages must continue to be transmitted by operators to the Mogadishu FIC via the AFTN using normal procedures;

- on notification of a contingency situation by CACAS, ICAO or the appropriate alternate authority of an adjacent FIR, the ATS authorities operating the ACCs of the adjacent FIRs will activate the contingency procedures in accordance with their respective Letters of Agreement.

- the adjacent ACCs responsible for aircraft entering and transiting the Mogadishu FIR must communicate to concerned ATS units not less than 30 minutes beforehand, the estimated time over the Mogadishu FIR boundary entry points;

- the adjacent ACCs responsible for aircraft entering the Mogadishu FIR will instruct pilots to maintain the last flight level assigned and speed (Mach number technique if applicable) or as per flight level scheme allocation in force while overflying the Mogadishu FIR;

- the adjacent ACCs responsible for aircraft entering the Mogadishu FIR will not authorize any change in flight level or speed (Mach number technique, if applicable) later than 10 minutes before the aircraft enters the Mogadishu FIR, except in the case specified in (h) below;

- the adjacent ACCs responsible for aircraft entering the Mogadishu FIR will inform all aircraft, prior to entering the Mogadishu FIR, that they must communicate with the next (downstream) ATC unit at least  10 minutes before the estimated time over the Mogadishu FIR boundary exit points, or as may be agreed by the accepting ATS unit downstream; and

- operators may also chose to route around the Mogadishu FIR, and the controlling authorities of the neighbouring FIRs concerned will provide alternative contingency routes as appropriate.

Note: 	ATS units should recognize that when closures of airspace or airports are promulgated, individual airlines might have different company requirements regarding alternative routing arrangements. ATS units should endeavour to accommodate such requests within the confines of safety rules and procedures.

Transition to contingency scheme

  - During times of uncertainty (severe weather, volcanic ash, reported seismic activity, etc) when airspace closure seems most likely, aircraft operators should be prepared for a possible change in routing while en-route, familiarization with the alternative routes outlined in this Contingency Plan, as well as those which may be promulgated by Somalia (CACAS) via NOTAM or other form of aeronautical information.

  - In the event of airspace closure that has not been promulgated, Mogadishu FIC and adjacent ATS units should, to the extent possible, broadcast to all aircraft under their jurisdiction, what airspace is being closed and to standby for further instructions.

  - ATS providers should recognize that when closure of airspaces or airports is promulgated, individual airlines might have different company requirements as to their alternative routings. ATS providers should be alert to respond to such requests by aircraft and react commensurate with safety.

  - If circumstances lead to the closure of the Mogadishu FIR and no contingency routes are available throughout that FIR, aircraft will be required to route around the Mogadishu FIR. As much warning as possible will be provided by CACAS in the event of the complete closure of Mogadishu FIR.

Adjustment of Coordination Requirements

  - CACAS and adjacent ATS providers concerned will review the effectiveness of current coordination requirements and procedures in light of contingency operations or airspace closure, and make any necessary adjustments to the Mogadishu Contingency Plan.

Public Health Emergencies (PHE)

  - Mogadishu FIC is designated as the coordinating ATS unit in the event of a Public Health Emergency (PHE).

  - In the event of a PHE, such as the outbreak of a contagious airborne disease or reported existence of airborne pathogens presenting a risk of disease outbreak, the affected ATS Unit in coordination with the FIC should verify with the Department of Ports & Health (DPH), Ministry of Health, the existence and nature of the PHE, as well as procedures to be followed in relation to the specific PHE. Based on the information, guidance and instruction acquired from the DPH, the affected ATS Unit and/or FIC (as guided by the FIC for the specific PHE) should as soon as practicable and using the most expeditious means of communication, provide to air operators information on the PHE, as well as procedures to followed based on guidance and/or instructions from the DPH.

  - In order to avoid miscommunication that may result in inappropriate reaction from the public including air operators, information provided by the DPH should be obtained in written form and relayed to air operators in written form. Where communication means do no enable relay of written text, the information shall be read verbatim. To minimize occupation of the aeronautical communication channels, the information, guidance and instructions applicable to air operators should whenever possible, be sent to air operators’ through their company offices or agents.

Note:

Focal points, contact details and agreed lines of communication with air operator are contained in local ATS procedures manual (or Local ATS Instructions (LATSI)) of each ATS Unit and letters of procedure for coordination with the air operators at the FIC and each aerodrome.

  - In taking initiatives in relation to information, guidance or instruction obtained from the DPH, ATS units should:

- coordinate with the AIS Section with the objective of issuing NOTAM/s providing necessary detailed information as determined by the DPH; and

- take appropriate action to ensure close coordination with aerodrome operators, in consideration that the aerodrome operator/s actions relating to the same PHE may overlap with those taken by ATS units, and that action taken by ATS and the operators impacts on aerodrome operations.

Note:

- Further guidance to ATS units is contained in the local ATS procedures manual (or Local ATS Instructions (LATSI)) of each ATS Unit and letter of procedure for coordination between ATS and each aerodrome operator.

- At aerodromes where PHE response committees have been established, the committee may be the source of information and guidance for various parties. In this case, a representative of the ATS authority participating in such a committee will be the focal point for communication and coordination between ATS units and the PHE response committee.

- While it is expected that entities involved in responding to PHEs provide contact list updates whenever there are changes in their organizations, it is also the responsibility of ATS units to ensure that contact details for PHE Focal Points are updated regularly.

# TRANSFER OF CONTROL AND COORDINATION OF TRAFFIC

6.1	The transfer of control and communication should be at the common FIR boundary between Mogadishu and its neighbouring States, unless there is mutual agreement between them. ATS providers should review current coordination requirements in light of contingency operations or notification at short notice of airspace closure.

# VOLCANIC ASH CONTINGENCY PLAN (VACP)

7.1	In the event of volcanic ash activity affecting the whole or part of the Mogadishu FIR, detailed contingency arrangements specifically related to volcanic ash are contained in Appendix G to this document.

# PILOT AND OPERATOR PROCEDURES

Filing of flight plans

  - Flight planning requirements for the Mogadishu FIR are to be followed in accordance with the ICAO the PANS-ATM (Doc 4444).

Overflight approval

  - In a contingency situation, flights may be re-routed at short notice and it may not be possible for operators to give the required advanced notice in a timely manner to obtain overflight approval. However, the current requirements and procedures for overflight approval of the Mogadishu FIR as provided for in Somalia NOTAM 0045/12 shall continue to be applicable.

  - With regard to other FIRs, aircraft operators are to obtain overflight approval from States responsible for such airspaces in accordance with the procedures and requirements of such States.

  - Coordination for special arrangements to expedite flight approvals for aircraft transiting the Mogadishu FIR in a contingency situation may be coordinated with CACAS and adjacent ATS units on a case by case basis, as addressed in the Letters of Agreement. Aircraft operators should note however that overflight approval remains the responsibility of the State whose territory is to be overflown.

Pilot operating procedures

  - Aircraft overflying the Mogadishu FIR shall follow the following procedures:

- all aircraft proceeding along the ATS routes established in this Contingency Plan will comply with the instrument flight rules (IFR) and will be assigned a flight level in accordance with the flight level allocation scheme (FLAS) applicable to the route(s) being flown as specified in Appendix C to this document;

- flights are to file flight plans using the Contingency Routes specified in Appendix C to this document, according to their airport of origin, routing and destination;

- pilots are to keep a continuous watch on the specified contingency radio frequencies as specified in the Letters of Agreement and transmit position information and estimates in accordance with normal ATC position reporting procedures using the English language;

- pilots are to maintain during their entire flight time within Mogadishu FIR, the flight level last assigned by the last ACC or ATS unit responsible for the provision of ATC service, prior to the aircraft entering the Mogadishu FIR. In the event that the last assigned flight level does not correspond to the flight level allocation scheme (FLAS) applicable to the Mogadishu Contingency Plan, the pilot should establish contact with the ATS unit responsible for the provision of service to clarify, and if unable, shall adjust to the FLAS as soon as possible once in the contingency airspace. The pilot shall, under no circumstances, change this level and Mach number, except in cases of emergency and for flight safety reasons. In addition, the last SSR transponder assigned shall be maintained or, if no transponder has been assigned, transmit on SSR code 2000;

- aircraft are to reach the flight level last assigned by the responsible ACC at least 10 minutes before entering the Mogadishu FIR or as otherwise instructed by the appropriate ATC unit in accordance with the Letters of Agreement;

- pilots are to include in their last position report prior to entering the Mogadishu FIR, the estimated time over the entry point of the Mogadishu FIR and the estimated time of arrival over the relevant exit points of the Mogadishu FIR;

- pilots are to contact the next adjacent ACC as soon as possible, and at the latest, 10 minutes before the estimated time of arrival over the FIR boundary exit points of Mogadishu FIR;

- whenever in-flight emergencies and/or flight safety reasons make it impossible to maintain the flight level assigned for transit of Mogadishu FIR, pilots are to climb or descend well to the right of the centerline of the contingency route, and if deviating outside the Mogadishu FIR, to immediately inform the ACC responsible for that airspace. Pilots are to make blind transmissions on 121.5 Mhz and 126.9 Mhz of the relevant emergency level change message, indicating the aircraft call sign, the aircraft position, the flight levels being vacated and crossed, etc);

- recognizant of the fact that not all operational circumstances can be addressed by this Contingency Plan, pilots are to maintain a high level of alertness when operating in the Mogadishu contingency airspace and take appropriate action to ensure safety of flight; and

- Pilots should maintain continuous listening watch on VHF emergency frequency 121.5 Mhz and IFBP frequency 126.9 Mhz at all times when operating in the Mogadishu contingency airspace.

Interception of civil aircraft

  - Pilots need to be aware that in light of current international circumstances, a contingency routing requiring aircraft to operate off of normal traffic flows, could result in an intercept by military aircraft. Aircraft operators must therefore be familiar with international intercept procedures contained in ICAO Annex 2 to the Chicago Convention –Rules of the Air, paragraph 3.8 and Appendix 2.

  - Should conditions prevailing in the airspace over the territory and territorial  waters of Somalia during contingency period result in the interception of civil aircraft by military aircraft, the pilot shall immediately take the following action:

- follow the instructions given by the intercepting aircraft, interpreting and responding to visual signals in accordance with international procedures;

- notify, if possible, the appropriate air traffic services unit;

- attempt to establish radio communication with the intercepting aircraft or with the appropriate intercept control unit, by making a general call on the emergency frequency 121.5 Mhz, giving the identity of the intercepted aircraft and the nature of the flight. If no contact has been established and if practicable, repeat this call on the emergency frequency 243 Mhz;

- if equipped with SSR transponder, select Mode A, Code 7700, unless otherwise instructed by the appropriate air traffic services unit;

- if equipped with ADS-B or ADS-C, select the appropriate emergency functionality, if available, unless otherwise instructed by the appropriate air traffic services unit;

- if any instructions received by radio from any sources conflict with those given by the intercepting aircraft by visual signals or by radio, the intercepted aircraft shall request immediate clarification while continuing to comply with the visual or radio instructions given by the intercepting aircraft.

Note: The above interception procedures are consistent with provisions contained in Annex 2 (Section 3.8) to the Chicago Convention.

Public Health Emergencies (PHE)

8.8	In the event of a PHE, such as the outbreak of a contagious airborne disease or existence of airborne pathogens presenting a high risk of disease outbreak, pilots should follow instructions provided by Mogadishu FIC, or as published through NOTAMs related to Mogadishu airspace and aerodromes. In the event that air operator company response requires pilots to take action that is necessarily different from that triggered by the DPH, concerned pilots or their air operator representative shall, without delay, provide relevant information to ATS for consideration and relay to the DPH.

Volcanic Ash Contingency Plan

8.9		In the event of volcanic ash activity affecting the whole or part of the Mogadishu FIR, pilots should follow instructions from Mogadishu FIC or as published my NOTAMs related to Mogadishu airspace. Detailed contingency arrangements specifically related to volcanic ash are contained in Appendix G to this document.

- COMMUNICATION PROCEDURES

Procedures for Reduced/Loss of Radio Communication

  - When operating within the contingency airspace of Mogadishu FIR, pilots should use normal radio communication procedures where ATS services are available or as otherwise notified by NOTAM.

9.2	If communication is lost on the normal ATS frequencies allocated, pilots should try the next applicable frequency, e.g. if en-route contact is lost then try the next appropriate handover frequency. It should be expected that loss of communication may be temporary. As such, if following the loss of communication pilots are still unable to establish two-way radio communication on other frequencies, pilots should consider periodic attempts on the frequency on which two-way radio communication was lost. In any case, in the absence of two-way communication with ATC, pilots should continue to make routine position reports on the appropriate frequencies, and broadcast positions on the specified contingency frequencies.

Communication frequencies

9.3			A list of frequencies to be used for the contingency routes and the ATS units providing flight information service (FIS) and air-ground communication monitoring for the Mogadishu FIR is detailed at Appendix F to this document.

10.	AERONAUTICAL INFORMATION SUPPORT SERVICES

Aeronautical Support Information Services (AIS)

10.1	NOTAM services will be used optimally to mitigate against loss of radio and other forms of communication in Mogadishu FIR. NOTAMs will be used as necessary to support coordination and operational procedures that would be established before, during and after any contingency phase.

10.2	In the event of unavailability of AIS services for Somalia (CACAS), NOTAM services will be provided by neighboring AIS authorities in accordance with Letters of Agreement.

Meteorological Services (MET)

10.3	It is expected that the Somalia (CACAS) MET services would continue to be available in the event of an ATS contingency situation. However, should ATS services for the Mogadishu FIR be withdrawn, timely MET information may not be immediately available to aircraft in flight. Alternative means of obtaining up to date MET information concerning the Mogadishu FIR will be provided to the extent possible through the adjacent ATS authorities.

11.	SEARCH AND RESCUE

Notification and Coordination

  - The SAR authority responsible for the Mogadishu SRR is:

Name:	Civil Aviation Caretaker Authority for Somalia (CACAS)

Address: P.O. Box 46294, 00100

Nairobi, Kenya

Tel: +254 20 762 2774 / 762 2785

Fax: +254 20 712 2340

AFTN: HCMMYAYX

E-mail:	icao.somalia@icao.unon.org

11.2		The SAR Point of Contact (SPOC) is:

Name: Mr. Wilson Owino

Tel: +254 20 762 2774 / 762 2785

Fax: +254 20 712 2340

AFTN: HCMMYAYX

E-mail: Wilson.owino@icao.unon.org

  - In the event that the Mogadishu FIC is not available to coordinate a given SAR operation, the responsibility for coordination with Djibouti, Ethiopia, Kenya, India, Seychelles and Yemen Rescue Coordination Centres (RCCs) for aircraft emergencies and incidents within the Mogadishu FIR will be undertaken by CACAS, in close coordination with the ICAO ESAF Regional Office in Nairobi.

- RESPONSIBILITY OF THE ACCREDITED ICAO REGIONAL OFFICE (ESAF)

12.1	The ICAO ESAF Regional Office which is accredited to the State of Somalia will:

- Closely monitor the situation and coordinate with all affected States and organizations including the IATA Regional Office, so as to ensure to the extend practical the continuity of air navigation and the provision of air navigation services to international air traffic in the AFI Region area of accreditation;

- Note any incidents reported and provide support to CACAS in taking appropriate action;

- Provide assistance as necessary on any issues with the Civil Aviation Administrations involved with and supporting the Mogadishu Contingency Plan; and

- Keep the President of the Council of ICAO, the Secretary General, Director Air Navigation Bureau, and Chief Air Traffic Management in Montreal continuously informed on developments, including activation and termination of the Mogadishu Contingency Plan.

APPENDIX A

Contact details for all concerned States, IATA and accredited ICAO Regional Office.

| State/ Organization | Point of contact | Telephone | E-mail |
| --- | --- | --- | --- |
| Djibouti | Djibouti APP/TWR | Tel: +253 342130 Tel: +253 340977 | twr@aeroport-jib.aero |
| Ethiopia | Addis ACC/FIC | Tel: +251 11 6611156/7 Tel: +251 11 6650519 | caa.eats@ethionet.et |
| Kenya | Nairobi ACC/FIC | Tel: +254 20827101 | jomo@kcaa.or.ke |
| India | Mumbai FIC | Tel: +912 226 828002 Tel: +912 226 828088 | Not available |
| Seychelles | Seychelles ACC/FIC | Tel: +248 384193 Tel: + 248 4384000 | atcc@scaa.sc |
| Yemen | Sana’a ACC/FIC | Tel: +967 1345914 Tel: +967 1344672 | anccns1@gmail.com |
| IATA | Tanja Grobotek | Tel : +271 15232714 | grobotek@iata.org |
| ICAO | Mr. M. Belayneh, Regional Director | Tel: +254 20 762 2395 Fax: +254 20 762 1092 | MBelayneh@icao.int |
|  | Mr. B Sekwati Deputy Regional Director | Tel: +254 20 762 2370 Fax: +254 20 762 1092 | BSekwati@icao.int |
|  | Mr. S M Machobane, RO ATM/SAR | Tel: +254 20 762 2372 Fax: +254 20 762 1092 | SMachobane@icao.int |
|  | Mr. D Labrosse RO ATM/SAR | Tel: +254 20 762 2368 Fax: +254 20 762 1092 | Dlabrosse@icao.int |

APPENDIX B

Contact details of the CCC members

| State/ Organization | Point of contact | Telephone | E-mail |
| --- | --- | --- | --- |
| CACAS | Mr. Kemoitse Mosupukwa Project Coordinador | Tel: +254 20 7622785/6 | Kemoitse.mosupukwa@icao.unon.org |
| CACAS | Mr. Wilson Owino Deputy Project Coordinador | Tel: +254 20 7622785/6 | Wilson.owino@icao.unon.org |
| CACAS | Mr. Humphrey Mwachoki FIC Supervisor | Tel: +254 20 7622785/6 | Humphrey.mwachoki@icao.unon.org |
| CACAS | Mr. Ali Jama Abdi ATC Shift Supervisor | Tel: +254 20 7622785/6 | Ali.jama@icao.unon.org |
| CACAS | Mr. Augustine Warratho Chief Meteorological Officer | Tel: +254 20 7622785/6 | Augustine.warratho@icao.unon.org |
| ICAO | Mr. Seboseso Machobane, RO ATM/SAR | Tel: +254 20 762 2372 Fax: +254 20 762 1092 | Smachobane@icao.int |
| ICAO | Mr. David Labrosse RO ATM/SAR | Tel: +254 20 762 2368 Fax: +254 20 762 1092 | Dlabrosse@icao.int |
| UNSOA | To be notified | To be notified | To be notified |

APPENDIX C

Contingency route structure during partial or total unavailability of the Mogadishu FIR

| Present ATS Route | Contingency Route Number | Contingency Routings | FLAS | Minimum Longitudinal Separation | FIRs Involved |
| --- | --- | --- | --- | --- | --- |
| UG450 | CR1 | KESOM-TULAP-MOGDU -KATHY-EMALU-AMPEX -NABAM-ORLID | Eastbound FL330 Westbound FL300 | 15 minutes | Nairobi Mumbai |
| UB400 | CR2 | ITMAR-RAGGS-MOGDU -BUBEM-VEDET | Eastbound FL290/310 Westbound FL360/380 | 15 minutes | Nairobi Sana’a |
| UM665 | CR3 | ITLOX-RAGGS-TULAP -MANDERA | Eastbound FL350/390/410 Westbound FL 280/340/400 | 15 minutes | Addis Ababa Seychelles |
| UN303/ UR775/ UR780 | CR4 | APKAK-MOGDU-SOLUL -ALNAB-HARGA-NAPGO | Eastbound FL350/390/410 Westbound FL280/340/400 | 15 minutes | Sana’a Seychelles |
| UB413 | CR5 | AVIMO-EMALU-EGTUL -DAROT-MERMI-EGROV -ZIZAN | Eastbound FL370/390 Westbound FL280/340/400 | 15 minutes | Sana’a Seychelles |
| UR401 | CR6 | AXINA-AMPEX-EPSIV -EVEBU-EKBEL-SUHIL | Eastbound FL350/410/450/ 490 Westbound FL340/360/400/ 430/470 | 15 minutes | Sana’a Seychelles |
| UR403 | CR7 | MUSBI-DAROT-AXIKU -BOMIX | Eastbound FL350 Westbound FL300/360 | 15 minutes | Addis Ababa Sana’a |
| UG657/ UA405/ UB312 | CR8 | ASKEN-HARGA-EGROV -PAKER | Eastbound FL310/330 Westbound FL300 | 15 minutes | Addis Ababa Sana’a |
| UG657/ UA405 UM651 | CR9 | ASKEN-HARGA-IMVEB -OKTOB | Eastbound FL310/330 Westbound FL320/380 | 15 minutes | Addis Ababa Sana’a |
| UG450/ UL303 | CR10 | KESOM-TULAP-MOGDU -ESTOK | Eastbound FL330 Westbound FL320 | 15 minutes | Seychelles |

---------------

APPENDIX D

APPENDIX E

SAMPLE NOTAMS

MOGADISHU FIR CONTINGENCY NOTAM

SCENARIO 1:	PARTIAL UNAVAILABILITY OF THE AIRSPACE

UNAVAILABILITY OF AIRSPACE IN MOGADISHU FIR WEST OF LONGITUDE 047 00 00 E

NOTAM  ……. .WEF …… TO ……….. AIRSPACE WEST OF LONGITUDE 047 00 00E UNAVAILABLE WITHIN THE MOGADISHU FLIGHT INFORMATION REGION. ALL FLIGHTS SHALL COMPLY WITH THE REQUIREMENT TO SELECT SPECIFIC CONTINGENCY ROUTES AND FLIGHT LEVELS APPLICABLE TO THE CONTINGENCY ROUTES AS DETAILED HERE BELOW. ADJACENT AREA CONTROL CENTRES OF SANAA, MUMBAI AND SEYCHELLES WILL ALLOCATE ONLY THE CONTINGENCY ROUTES AND FLIGHT LEVELS SPECIFIED AS FOLLOWS:

A) 	CR5 (UB413) ZIZAN-EGROV-MERMI-DAROT-EGTUL-EMALU-AVIMO: EASTBOUND FL370/390 WESTBOUND FL 280/340/400. MINIMUM LONGITUDINALSEPARATION APPLICABLE IS 15 MINUTES.

B) 	CR6 (UR401) AXINA- AMPEX-EPSIV-EVEBU-EKBEL-SUHIL: EASTBOUND FL350/410 WESTBOUND FL340/360/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

PILOTS WHO HAVE BEEN ASSIGNED WITH A FLIGHT LEVEL NOT IN ACCORDANCE WITH THE FLAS, SHOULD TRY TO ESTABLISH CONTACT WITH THE ATS UNIT RESPONSIBLE FOR THE PROVISION OF SERVICE TO CLARIFY, AND IF UNABLE, ADJUST TO THE FLAS AS SOON AS POSSIBLE ONCE IN THE CONTINGENCY AIRSPACE.

================================================

SCENARIO 2:	PARTIAL UNAVAILABILITY OF THE AIRSPACE

UNAVAILABILITY OF AIRSPACE IN MOGADISHU FIR EAST OF LONGITUDE 047 00 00 E

NOTAM  ……. .WEF …… TO ……….. AIRSPACE EAST OF LONGITUDE 047 00 00E UNAVAILABLE WITHIN THE MOGADISHU FLIGHT INFORMATION REGION. ALL FLIGHTS SHALL COMPLY WITH THE REQUIREMENT TO SELECT SPECIFIC CONTINGENCY ROUTES AND FLIGHT LEVELS APPLICABLE TO THE CONTINGENCY ROUTES AS DETAILED HEREBELOW. ADJACENT AREA CONTROL CENTRES OF NAIROBI, ADDIS ABABA, SANAA AND SEYCHELLES WILL ALLOCATE ONLY THE CONTINGENCY ROUTES AND FLIGHT LEVELS SPECIFIED AS FOLLOWS:

- CR3 (UM665) ITLOX- RAGGS-TULAP-MANDERA (MAV):  EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR4 (UN303/UR775/UR780) APKAK-MOGUD-SOLUL-ALNAB-HARGA-NAPGO: EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR8 (UG657/UA405/UB312) ASKEN-HARGA-EGROV-PAKER: EASTBOUND FL310/330 WESTBOUND FL300. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR9 (UG657/UA405/UM651) ASKEN-HARGA-IMVEB-OKTOB: EASTBOUND FL310/330 WESTBOUND FL320/380. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

PILOTS WHO HAVE BEEN ASSIGNED WITH A FLIGHT LEVEL NOT IN ACCORDANCE WITH THE FLAS, SHOULD TRY TO ESTABLISH CONTACT WITH THE ATS UNIT RESPONSIBLE FOR THE PROVISION OF SERVICE TO CLARIFY, AND IF UNABLE, ADJUST TO THE FLAS AS SOON AS POSSIBLE ONCE IN THE CONTINGENCY AIRSPACE.

================================================

SCENARIO 3: 	PARTIAL UNAVAILABILITY OF THE AIRSPACE

UNAVAILABILITY OF AIRSPACE IN MOGADISHU FIR SOUTH OF LATITUDE 07 00 00N

NOTAM  ……. .WEF …… TO ……….. AIRSPACE SOUTH OF LATITUDE 07 00 00N UNAVAILABLE WITHIN THE MOGADISHU FLIGHT INFORMATION REGION. ALL AIRCRAFT SHALL COMPLY WITH THE REQUIREMENT TO SELECT SPECIFIC CONTINGENCY ROUTES AND FLIGHT LEVELS APPLICABLE TO THE CONTINGENCY ROUTES AS DETAILED HEREBELOW. ADJACENT AREA CONTROL CENTRES OF ADDIS ABABA, SANAA, MUMBAI AND SEYCHELLES WILL ALLOCATE ONLY THE CONTINGENCY ROUTES AND FLIGHT LEVELS SPECIFIED AS FOLLOWS:

- CR6 (UR401) AXINA- AMPEX-EPSIV-EVEBU-EKBEL-SUHIL: EASTBOUND FL350/410 WESTBOUND FL340/360/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR7 (UR403) MUSBI-DAROT-AXIKU-BOMIX: EASTBOUND FL350 WESTBOUND FL300/360. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR8 (UG657/UA405/UB312) ASKEN-HARGA-EGROV-PAKER: EASTBOUND FL310/330 WESTBOUND FL300. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR9 (UG657/UA405/UM651) ASKEN-HARGA-IMVEB-OKTOB: EASTBOUND FL310/330 WESTBOUND FL320/380. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

PILOTS WHO HAVE BEEN ASSIGNED WITH A FLIGHT LEVEL NOT IN ACCORDANCE WITH THE FLAS, SHOULD TRY TO ESTABLISH CONTACT WITH THE ATS UNIT RESPONSIBLE FOR THE PROVISION OF SERVICE TO CLARIFY, AND IF UNABLE, ADJUST TO THE FLAS AS SOON AS POSSIBLE ONCE IN THE CONTINGENCY AIRSPACE.

================================================

SCENARIO 4: 	PARTIAL UNAVAILABILITY OF THE AIRSPACE

UNAVAILABILITY OF AIRSPACE IN MOGADISHU FIR NORTH OF LATITUDE 07 00 00N

NOTAM  ……. .WEF …… TO ……….. AIRSPACE NORTH OF LATITUDE 07 00 00N UNAVAILABLE WITHIN THE MOGADISHU FLIGHT INFORMATION REGION. ALL AIRCRAFT SHALL COMPLY WITH THE REQUIREMENT TO SELECT SPECIFIC CONTINGENCY ROUTES AND FLIGHT LEVELS APPLICABLE TO THE CONTINGENCY ROUTES AS DETAILED HEREBELOW. ADJACENT AREA CONTROL CENTRES OF NAIROBI, SANAA AND SEYCHELLES WILL ALLOCATE ONLY THE CONTINGENCY ROUTES AND FLIGHT LEVELS SPECIFIED AS FOLLOWS:

- CR3 (UM665) ITLOX- RAGGS-TULAP-MANDERA (MAV):  EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR4 (UN303/UR775/UR780) SOLUL-MOGDU-APKAK: EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR10 (UG450/UL303) KESOM-TULAP-MOGDU-ESTOK: EASTBOUND FL330 WESTBOUND FL320. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

PILOTS WHO HAVE BEEN ASSIGNED WITH A FLIGHT LEVEL NOT IN ACCORDANCE WITH THE FLAS, SHOULD TRY TO ESTABLISH CONTACT WITH THE ATS UNIT RESPONSIBLE FOR THE PROVISION OF SERVICE TO CLARIFY, AND IF UNABLE, ADJUST TO THE FLAS AS SOON AS POSSIBLE ONCE IN THE CONTINGENCY AIRSPACE.

================================================

SCENARIO 5: UNAVAIBILITY OF ATS IN MOGADISHU FIR

NOTAM  ……..WEF …… TO ……….. MOGADISHU FIC TEMPORARILY UNABLE TO PROVIDE AIR TRAFFIC SERVICE IN THE ENTIRE AIRSPACE WITHIN THE MOGADISHU FIR. ALL AIRCRAFT SHALL COMPLY WITH REQUIREMENT TO SELECT SPECIFIC CONTINGENCY ROUTES AND FLIGHT LEVELS APPLICABLE TO THE CONTINGENCY ROUTES IN ACCORDANCE WITH THE FLIGHT LEVEL ALLOCATION SCHEME (FLAS) DETAILED HEREBELOW. ADJACENT AREA CONTROL CENTRES OF NAIROBI, ADDIS ABABA, SANAA, MUMBAI AND SEYCHELLES WILL ALLOCATE ONLY THE CONTINGENCY ROUTES AND FLIGHT LEVELS SPECIFIED AS FOLLOWS:

- CR1 (UG450) KESOM-TULAP-MOGDU-KATHY-EMALU-AMPEX-NABAM-ORLID: EASTBOUND FL330 WESTBOUND FL320. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR2 (UB400) ITMAR-RAGGS-MOGDU-BUBEN-VEDET: EASTBOUND FL290/310 WESTBOUND FL360/380. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR3 (UM665) ITLOX- RAGGS-TULAP-MANDERA (MAV):  EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR4 (UN303/UR775/UR780) APKAK-MOGDU-SOLUL-ALNAB-HARGA-NAPGO: EASTBOUND FL350/390/410 WESTBOUND FL280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR5 (UB413) AVIMO-EMALU-EGTUL-DAROT-MERMI-EGROV-ZIZAN: EASTBOUND FL370/390 WESTBOUND FL 280/340/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR6 (UR401) AXINA- AMPEX-EPSIV-EVEBU-EKBEL-SUHIL: EASTBOUND FL350/410 WESTBOUND FL340/360/400. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR7 (UR403) MUSBI-DAROT-AXIKU-BOMIX: EASTBOUND FL350 WESTBOUND FL300/360. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR8 (UG657/UA405/UB312) ASKEN-HARGA-EGROV-PAKER: EASTBOUND FL310/330 WESTBOUND FL300. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR9 (UG657/UA405/UM651) ASKEN-HARGA-IMVEB-OKTOB: EASTBOUND FL310/330 WESTBOUND FL320/380. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

- CR10 (UG450/UL303) KESOM-TULAP-MOGDU-ESTOK: EASTBOUND FL330 WESTBOUND FL320. MINIMUM LONGITUDINAL SEPARATION APPLICABLE IS 15 MINUTES.

PILOTS WHO HAVE BEEN ASSIGNED WITH A FLIGHT LEVEL NOT IN ACCORDANCE WITH THE FLAS, SHOULD TRY TO ESTABLISH CONTACT WITH THE ATS UNIT RESPONSIBLE FOR THE PROVISION OF SERVICE TO CLARIFY, AND IF UNABLE, ADJUST TO THE FLAS AS SOON AS POSSIBLE ONCE IN THE CONTINGENCY AIRSPACE.

APPENDIX F

List of Frequencies to be used

1.	A list of frequencies to be used for the contingency routes and the ATS units providing flight information service (FIS) and air-ground communication monitoring for the Mogadishu FIR is as follows:

Mogadishu Flight Information Centre (FIC)

- Day: HF 11300 Khz or 13288 Khz

- Night: HF 11300 Khz or 5517 Khz

2.	In the event of these frequencies are not being available, aircraft are to contact Seychelles Control on the following frequencies:

Seychelles Area Control Centre (ACC)

- Day: HF 11300 Khz, 8879 Khz

- HF 11300 Khz, 5634 Khz, 3476 Khz, 8879 Khz

---END---

Appendix G

Volcanic Ash Contingency Plan (VACP