| TRINIDAD AND TOBAGO ASBU Air Navigation Reporting Form (ANRF) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIA |  | 1 | Block - Module | B0 - APTA | Date |  | 30th NOVEMBER, 2016 |  |
| Module Description: The use of Performance-based Navigation (PBN) and ground-based augmentation system (GBAS) landing system (GLS) procedures to enhance the reliability and predictability of approaches to runways, thus increasing safety, accessibility and efficiency. This is possible through the application of basic global navigation satellite system (GNSS), Baro-vertical navigation (VNAV), satellite-based augmentation system (SBAS) and GLS. The flexibility inherent in PBN approach design can be exploited to increase runway capacity. |  |  |  |  |  |  |  |  |
| Element Implementation Status |  |  |  |  |  |  |  |  |
| 1 | Element Description: PBN Approach Procedures with vertical guidance (LPV, LNAV/VNAV minima, using SBAS and Baro VNAV) |  |  |  |  | Date Planned/Implemented JAN 2017 |  | Status To be Implemented |
|  | Status Details TTPP – LNAV Approaches for both runway ends (10/28) are implemented. BARO-VNAV Approaches to be implemented by 5 JAN 2017. TTCP – LNAV Approaches for both runway ends (11/29) are implemented. BARO-VNAV Approaches to be implemented by 5 JAN 2017. *Final stages of approach charting for publication. AIRAC date 10 NOV 2016. |  |  |  |  |  |  |  |
| 2 | Element Description: PBN Approach Procedures without vertical guidance (LP, LNAV minima; using SBAS) |  |  |  |  | Date Planned/Implemented OCTOBER 2003 |  | Status Implemented |
|  | Status Details No SBAS infrastructure at TTPP/TTCP. TTPP – LNAV Approaches for both runway ends (10/28) are implemented. TTCP – LNAV Approaches for both runway ends (11/29) are implemented. |  |  |  |  |  |  |  |
| 3 | Element Description: GBAS Landing System (GLS) Approach procedures |  |  |  |  | Date Planned/Implemented N/A |  | Status N/A |
|  | Status Details At this point in time this is not required |  |  |  |  |  |  |  |
| Achieved Benefits |  |  |  |  |  |  |  |  |
| Access and Equity Element 1&2: Increased aerodrome accessibility, especially around mountains and in low visibility operating conditions |  |  |  |  |  |  |  |  |
| Capacity Element 1&2: Increased runway capacity |  |  |  |  |  |  |  |  |
| Efficiency Element 1&2: Reduced fuel burn due to lower minima, fewer diversions, cancellations, delays |  |  |  |  |  |  |  |  |
| Environment Element 1&2: Reduced emissions due to reduced fuel burn due to lowering minima for landing that result in fewer diversions, cancellations, and/or delays. |  |  |  |  |  |  |  |  |
| Safety Element 1&2: Increased safety through stabilized approach paths. |  |  |  |  |  |  |  |  |
| Implementation Challenges |  |  |  |  |  |  |  |  |
| Ground system Implementation None |  |  |  |  |  |  |  |  |
| Avionics Implementation None |  |  |  |  |  |  |  |  |
| Procedures Availability None |  |  |  |  |  |  |  |  |
| Operational Approvals None |  |  |  |  |  |  |  |  |
| Notes None |  |  |  |  |  |  |  |  |