| TRINIDAD AND TOBAGO ASBU Air Navigation Reporting Form (ANRF) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIA |  | 3 | Block - Module | B0 - NOPS | Date |  | 30th NOVEMBER,2016 |  |
| Module Description: Air traffic flow management (ATFM) is used to manage the flow of traffic in a way that minimizes delays and maximizes the use of the entire airspace. ATFM can regulate traffic flows involving departure slots, smooth flows and manage rates of entry into airspace along traffic axes, manage arrival time at waypoints or flight information region (FIR)/sector boundaries and re-route traffic to avoid saturated areas. ATFM may also be used to address system disruptions including a crisis caused by human or natural phenomena. |  |  |  |  |  |  |  |  |
| Element Implementation Status |  |  |  |  |  |  |  |  |
| 1 | Element Description: ATFM |  |  |  |  | Date Planned/Implemented December 2016 |  | Status To be implemented |
|  | Status Details ATFM procedures utilized in the Piarco FIR. An ATFM system will be in place by December 31, 2016 |  |  |  |  |  |  |  |
| Achieved Benefits |  |  |  |  |  |  |  |  |
| Access and Equity Improved access by avoiding disruption of air traffic in periods of demand higher than capacity. ATFM processes take care of equitable distribution of delays. |  |  |  |  |  |  |  |  |
| Capacity Better utilization of available capacity, network-wide; in particular the trust of ATC not being faced by surprise to saturation tends to let it declare/use increased capacity levels; ability to anticipate difficult situations and mitigate them in advance. |  |  |  |  |  |  |  |  |
| Efficiency Reduced fuel burn due to better anticipation of flow issues; a positive effect to reduce the impact of inefficiencies in the ATM system or to dimension it at a size that would not always justify its costs (balance between cost of delays and cost of unused capacity). Reduced block times and times with engines on. |  |  |  |  |  |  |  |  |
| Environment Reduced fuel burn as delays are absorbed on the ground, with shut engines; re-routing, however, generally put flights on a longer distance, but this is generally compensated by other airline operational benefits. |  |  |  |  |  |  |  |  |
| Safety Reduced occurrences of undesired sector overloads. |  |  |  |  |  |  |  |  |
| Implementation Challenges |  |  |  |  |  |  |  |  |
| Ground system Implementation None |  |  |  |  |  |  |  |  |
| Avionics Implementation None |  |  |  |  |  |  |  |  |
| Procedures Availability None |  |  |  |  |  |  |  |  |
| Operational Approvals None |  |  |  |  |  |  |  |  |
| Notes None |  |  |  |  |  |  |  |  |