| TRINIDAD AND TOBAGO ASBU Air Navigation Reporting Form (ANRF) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIA |  | 4 | Block - Module | B0 - TBO | Date |  | 30th NOVEMBER, 2016 |  |
| Module Description: Implements an initial set of data link applications for surveillance and communications in air traffic control (ATC), supporting flexible routing, reduced separation and improved safety. |  |  |  |  |  |  |  |  |
| Element Implementation Status |  |  |  |  |  |  |  |  |
| 1 | Element Description: ADS-C over oceanic and remote areas |  |  |  |  | Date Planned/Implemented Implemented July 7th 2016 |  | Status Compliant |
|  | Status Details ADC-C is used for the communication over the Atlantic ocean. |  |  |  |  |  |  |  |
| 2 | Element Description: Continental CPDLC |  |  |  |  | Date Planned/Implemented Implemented July 7th 2016 |  | Status Compliant |
|  | Status Details CPDLC is used for the Oceanic operation |  |  |  |  |  |  |  |
| Achieved Benefits |  |  |  |  |  |  |  |  |
| Access and Equity |  |  |  |  |  |  |  |  |
| Capacity Element 1: A better localization of traffic and reduced separations allow the increasing of offered capacity. Element 2: Reduced communication workload and better organization of controller tasks allowing increased sector capacity. |  |  |  |  |  |  |  |  |
| Efficiency Element 1: Routes/tracks and flights can be separated by reduced minima, allowing flexible routings and vertical profiles closer to the user-preferred routes/tracks. |  |  |  |  |  |  |  |  |
| Environment Element 1: Reduced emissions as a result of reduced fuel burn |  |  |  |  |  |  |  |  |
| Safety Element 1: Increased situational awareness; ADS-C based safety nets like cleared level adherence monitoring, route adherence monitoring, danger area infringement warning; and better support to search and rescue. Element 2: Increased situational awareness; reduced occurrences of misunderstandings; solution to stuck microphone situations. |  |  |  |  |  |  |  |  |
| Implementation Challenges |  |  |  |  |  |  |  |  |
| Ground system Implementation |  |  |  |  |  |  |  |  |
| Avionics Implementation |  |  |  |  |  |  |  |  |
| Procedures Availability |  |  |  |  |  |  |  |  |
| Operational Approvals |  |  |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |  |