Exercice

Un crew captain at arrival reported on inconstancy of the homing figure provided by VOR SX 114.6 MHZ from the new station of Ory (2°23’65 W, 3°34’87N) distant of 50NM from the station of Badly located beyond the border with Neighborland State and operating on the same freqiuency.

The dysfunction is due to severe interference in the overlapping coverage of the two Stations.

# Exercise N°1: Notification of Deficiencies

| State | WACAF State English |  |
| --- | --- | --- |
| Minimum reporting area |  |  |
| Technical Area |  | CNS |
| Reference |  | AFI 7, Rec. 10/2 |
| Description |  | Geographical separation criteria for VOR and /or VOR/DME installations in the AFI Region |
| Deficiency 1 |  |  |
| Description |  | Inconstancy of the homing figure provided by VOR SX 114.6 MHZ from the new station of Ory (2°23’65 W, 3°34’87N) distant of 50NM from the station of Badly located beyond the border with Neighborland State and operating on the same frequency. |
| Rational for non-elimination |  |  |
| Priority |  | U |
| Remarks |  | Interference due to overlapping which would lead to misguidance in pilots homing on the targeted VOR or Aerodrome |

# Exercise N°2: Development and Submission of State Action Plan

| Corrective actions of Deficiency 1 |  |  |  |
| --- | --- | --- | --- |
| Actions | Responsible Bodies | Due date | Remarks |
| Issue a NOTAM | ANSP Badly, WACAF State English AIM Office | 25/04/24 | Caution airspace users on arrival at Aerodrome Badly |
| Turn off VOR Ory (2°23’65 W, 3°34’87N) | ANSP Badly | 26/04/24 |  |
| Notify Regional Office through the state for advice | ANSP Badly, WACAF State English CAA | 27/04/24 | To expect feedback in two weeks period from 27/04/24 |
| Contact installer of VOR Ory on interference issues | ANSP Badly | 29/04/24 | To expect feedback from installer within one week from 29/04/24 |
| Discuss and agree on terms of reference for the relocation of VOR Ory | ANSP Badly, Installer | 13/05/24 | To complete discussions within one week from 6/04/24 |
| Contact and negotiate with Flight calibration service company | ANSP Badly | 15/05/24 | Coordinate between Flight calibration company and Installer |
| Relocate VOR Ory | Installer | 30/06/24 | Monitor NOTAM and advice airmen to caution on use of Badly Airspace |
| Flight check VOR Ory | ANSP Badly, Installer | 1/07/24 | Publish additional NOTAM for flight check exercise |
| Issue NOTAM on serviceability of VOR Ory | ANSP, WACAF State English AIM Office | 2/07/24 |  |
| Inform RO of the serviceability of the VOR Ory | WACAF State English CAA | 3/07/24 |  |

# Exercise N°3: Implementation of the action Plan and provision of evidence

# Exercise N°4: Development and Submission of APIRG Action plan

# Exercise N°5 : Implementation of Action plan and Submission of evidences

# Exercise N°6: Closing of deficiencies